SECTION 09624-Wood and Synthetic Athletic Flooring

PART 1 – GENERAL

  - DESCRIPTION

- Scope

- Furnish and install the AacerFlex TP with MP Sport Combination Resilient Athletic Floor System by Aacer Flooring.

- Related work specified under other sections.

- CONCRETE SUBFLOORS – SECTION 03___

- Slab shall be recessed to the depth of:

  - 2-1/2” using 25/32” Flooring using 3/4" Tri-Power Pads

  - 2-3/4” using 33/32” Flooring using 3/4" Tri-Power Pads

- The general contractor shall furnish and install the concrete subfloor depressing the slab sufficiently to accommodate the floor system.  The slab shall be steel troweled smooth to a tolerance of 1/8” in any 10’ radius by the general contractor.  High spots shall be ground level, and low spots filled in with approved leveling compound by the general contractor to the full approval of the installer.

- No curing compounds, agents or sealers shall be used.

- MEMBRANE WATERPROOFING-SECTION 07____

  - Concrete subfloors on or below grade shall be adequately waterproofed beneath the slab and at the perimeter walls and on earth side of below grade walls by general contractor using suitable type membrane.

- THRESHOLDS – SECTION 08___

- SLEEVES AND STANDARD INSERTS – SECTION 11___

  - REFERENCES

- DIN – Performance Standard 18032 Part 2

  - QUALITY ASSURANCE

- Manufacturer

- Manufacturer of resilient flooring shall be a firm specializing in manufacturing products specified in this section.

- Basis of design shall be “AacerFlex TP with Aacer MP Sport™ Combination System” sports floor system as provided by Aacer Flooring. (877) 582-1181, www.Aacerflooring.com.

- Materials other than those listed must be approved 15 days prior by written addendum.

Materials from non-approved manufacturers will not be accepted.

- Installer

- The installation of the floor system described in these specifications shall be completed by a firm familiar with the requirements of the system specified and fully experienced in procedures required for installing athletic flooring manufactured by Aacer Flooring.

- Installer shall be liable for all matters related to installation for a period of one year after the floor has been substantially installed and completed.

- Installer must have Aacer installation accreditation.

- Performance Testing

- Flooring system shall have been independently tested to the International Standards: DIN 18032, Part 2 or EN 14904.

- Independent DIN testing laboratory must test to all the required standards of the DIN testing methodologies.

- Independent DIN testing laboratory shall have Scientific Body Membership in the International Association of Sports Surface Sciences (ISSS).

  - SUBMITTALS

- Specification - Submit Aacer Flooring specification sheets and shop drawings as required.

- Sample - Submit required number of samples and color charts for system and gamelines of the specified system as requested by the owner/architect.

  - WORKING CONDITIONS

- The flooring and its components specified herein shall not be delivered or installed until all wet trades and overhead work is completed.  This includes all masonry, painting, plaster, tile, marble, and terrazzo, as well as all overhead mechanical trades.  The building shall be fully enclosed and weather tight and all permanent windows and doorways shall be installed.  Permanent heating and air conditioning shall be installed and working in accordance with building occupation requirements.

- During and after installation, building HVAC systems shall maintain a temperature and humidity range compatible with the expected high and low moisture content range of the flooring. The floor installer, based on the building’s HVAC control and geographical situation, shall determine this range.

- Environmental Limitations:

- Adhere to all MSDS requirements for materials employed in the work.  Protect all persons from exposure to hazardous materials at all times.

  - WARRANTY AND DISCLAIMER

- Aacer Flooring of Peshtigo, WI, hereby warrants the materials it has supplied to be free from manufacturing defects for a period of one year.  This warranty is in lieu of and excludes all other warranties expressed or implied including any implied warranties of merchantability or fitness for a particular purpose.

- The jobsite documentation forms by the flooring contractor shall become a part of the warranty and both the owner and flooring contractor shall retain record of said forms as a permanent reference for any abrogation.

- Flooring contractor warrants the install of the floor systems to be free from defects in materials and workmanship for a period of one year.

- Notification of claim shall be made within 30 days of discovery.

- It is the policy of Aacer Flooring to continuously improve its line of products. Therefore, Aacer Flooring reserves the right to change, modify, or discontinue systems, specifications and accessories of all products at any time without notice or obligation to purchaser.

PART 2 PRODUCTS

  - MATERIALS

- Vapor Barrier – 6 mil polyethylene

- Resilient Pads-

- Aacer Performance pad, 3/4” Tri Power pad, Orange 60 durometer.

- Optional pads (Specify or Delete)

  - Aacer Tri-Power® pad, 3/4" EPDM, red 70 durometer

  - Aacer Tri-Power® pad, 3/4" EPDM, blue 50 durometer(aerobics pad)

  - Aacer Tri-Power® pad, 1/2" (12mm), EPDM, green 60 durometer.

- Subfloor –

- 2 layers of 15/32” APA rated plywood sheathing.  Exposure 1

- 1 layer of 3/8” ULD grade plywood sheathing.

- MP Sport Adhesive

- Two component polyurethane adhesive, specially formulated for adhering recycled rubber sheet goods.

- MP Sport Underlayment

- Prefabricated rubber sheet material consisting of recycled rubber granules bound together with urethane binder as supplied by Aacer Flooring.

- Available thickness:

  - 8mm

- MP Sport Seal Coat

- Two component, thixotropic polyurethane material specially formulated for use as a pore sealer over recycled rubber underlayments.

- This material shall contain no mercury, lead, nor any heavy metals, PCB, or formaldehyde and be non solvent based.

- MP Sport Body Coat

- Two component 2mm self leveling polyurethane material specially formulated for use in sports floors.

- This material will contain no mercury, lead or any heavy metals, PCB, or formaldehyde.

- MP Sport Top Coat

- Elastic, two component polyurethane based coating specially formulated for use in athletic flooring systems.

- Specify system color(s)- in the standard color line (Refer to Aacer color chart)

- Fasteners –

- Subfloor – 1” coated staples.

- Subfloor – Screws and pl 400 adhesive

- Wall Base - Heavy duty, molded, vented cove base with pre-molded outside corners.

- MP Sport Game Line Paint

- Elastic, two component polyurethane coating specially formulated and pigmented for use as line paint in the above described systems.  Specify Aacer game line paint.

- Standard Colors- Black, White, Red, Green, Blue, Orange, and Yellow.  Refer to Aacer Color Chart.

PART 3 - EXECUTION

3.1. PRE-INSTALLATION INSPECTION

- Floor installer shall verify slab tolerance and moisture content of concrete.  Slab should be at less than 4 pounds vapor emission.

- Room shall be broom cleaned and free of any foreign debris.  Floor must be free of paint, grease, oil and all contaminants that will adversely affect proper adhesion to the substrate.

- Floor installer shall document site and working conditions prior to and during installation.  This documentation shall become a part of any warranty and may or may not affect fulfillment of any warranty.

- Report all discrepancies to the general contractor, construction manager or owner.

3.2. INSTALLATION

- SUBFLOOR –

- Cover entire slab with 6 mil polyethylene, sealing and lapping joints a minimum of 6”.

- AacerFlex panels shall be laid perpendicular to finished flooring.  (Optional laid at 45 degree angle)  Allow ¼” space between ends of abutted panels and stagger joints 4’.  Aacer Tri Power pads shall be installed on underside of bottom layer 12” on center, 32 per sheet.  Install solid blocking under bleachers in the stacked position.  Install stop blocking under bleachers in the extended position or heavy load areas as required.  Provide 1 ½” expansion void at perimeter and all vertical obstructions.

- Install second layer of plywood material laid diagonally at 45 degree angles over the first layer with 1/4" spacing on all edges, breaking rows at 4’.  Second layer shall be attached using 1” coated staples or equivalent, at a minimum of 12” on center.   If laying first layer at 45 degree angle, lay second layer at opposite 45 degree angle.

- Install third layer of plywood material laid diagonally at 45 degree angles over the first layer with 1/8" spacing on all edges, breaking rows at 4’.  Second layer shall be attached using 1” coated screws or equivalent, at a minimum of 12” on center with layer of PL 400 adhesive.   If laying first layer at 45 degree angle, lay second layer at opposite 45 degree angle Provide 1-1/2” expansion voids at perimeter and all vertical obstructions.  Install solid blocking at doorways, under bleachers in the stacked position, and below portable goals.

- UNDERLAYMENT-

- Thoroughly mix the MP Sport Underlayment Adhesive and apply to the concrete with a notched trowel or adhesive spatula.

- Roll the MP Sport Underlayment into the adhesive.  Leave a 3/8” gap at all walls and transitions.  Do not allow the sides of the underlayment to be in compression.  Allow a loose fit at the sides, even to 1/8” gap between rolls.

- Roll the entire surface of the underlayment with a 75 or 100 pound floor roller several times during the first 4 hours after installation.

- Check the mat for defects and perform repairs as necessary to achieve a smooth surface.

- SEAL COAT-

- Properly mix the MP Sport Seal Coat and apply to the underlayment with a smooth steel trowel.

- Fill all surface pores in the underlayment and all voids at the walls, doors, and penetrations.

- Allow seal coat to cure 12 hours before proceeding to the next step

- BODY COAT-

- Properly mix the MP Sport Body Coat and apply to the sealed underlayment in 2 applications with a Swedish Knife supplied by Aacer to a total thickness of 2mm.

- The application rate is 0.55 lbs/SF.

- Apply the material “wet-to-wet” to allow the material to flow and eliminate ridges.

- Allow the body coat to cure 12 hours before proceeding to the next step.

- Top Coat

- Properly mix the MP Sport Top Coat and apply to the Body Coat with an 18” to 24” low lint paint roller at a rate of 0.04 lbs./SF (550 SF/unit).  May be sprayed but requires 30% more material.

- Gamelines-

- Lay out gamelines with a high quality 1” masking tape.  No blue tape.

- Apply the MP Sport Gameline Paint with a paint roller.

- Pull the tape while the gameline coating is still wet/tacky.

- After the gamelines are completed (including graphics and borders), the area must be kept free of foot traffic for a minimum of 7 days.

3.3 FINISHING

- Remove excess debris and waste material from the work area.

- General Contractor shall lock floor area after floor is finished to allow proper curing time.

- BASE INSTALLATION

- Install vent cove base with cove base adhesive and/or mechanical attachment to wall.  Use pre-molded outside corners and mitered inside corners.

END OF SECTION 09642

AacerFlex TP with Aacer MP Sport Combination Spec sheet

December 2011

Revision level A