SECTION 09642-Wood Gymnasium Flooring

PART 1 – GENERAL

  - DESCRIPTION

- Related work specified under other sections.

- CONCRETE SUBFLOORS – SECTION 03___

- Slab depression is: 2 5/8”

- The general contractor shall furnish and install the concrete subfloor depressing the slab sufficiently to accommodate the floor system.  The slab shall be steel troweled smooth to a tolerance of 1/8” in any 10’ radius by the general contractor.  High spots shall be ground level, and low spots filled in with approved leveling compound by the general contractor to the full approval of the flooring contractor.

- MEMBRANE WATERPROOFING-SECTION 07____

  - Concrete subfloors on or below grade shall be adequately waterproofed beneath the slab and at the perimeter walls and on earth side of below grade walls by general contractor using suitable type membrane.

- THRESHOLDS – SECTION 08___

- SLEEVES AND STANDARD INSERTS – SECTION 11___

  - REFERENCES

- DIN – Performance Standard 18032 Part 2

- EN – Performance Standard 14904 World Standard

- FSC – Forest Stewardship Council™

- LEED – Leadership in Energy and Environmental Design

- MFMA – Maple Flooring Manufacturers Association

  - QUALITY ASSURANCE

- Manufacturer

- Manufacturer of resilient flooring shall be a firm specializing in manufacturing products specified in this section.

- Basis of design shall be “UltraFlex™ DC” sports floor system as provided by Aacer Flooring.  (877) 582-1181, www.Aacerflooring.com.

- Materials other than those listed must be approved 10 days prior by written addendum. Materials from non-approved manufacturers will not be accepted.

- Installer

- The installation of the floor system described in these specifications shall be completed by a firm familiar with the requirements of the system specified and fully experienced in procedures required for installing athletic flooring manufactured by Aacer Flooring.

- Flooring Contractor shall be liable for all matters related to installation for a period of one year after the floor has been substantially installed and completed.

- Installer must have Aacer installation accreditation.

  - Optional: MFMA accreditation.(Specify or Delete)

- Performance Testing

- Flooring system shall have been independently tested to the International Standards: DIN 18032, Part 2 or EN 14904.

- Independent DIN testing laboratory must be recognized by the MFMA and test to the required standards of the DIN testing methodologies.

- Independent DIN testing laboratory shall have Scientific Body Membership in the International Association of Sports Surface Sciences (ISSS).

  - SUBMITTALS

- Specification - Submit Aacer Flooring specification sheets and shop drawings as required.

- Sample - Submit required number of samples of the specified system as requested by the owner/architect.

- Maintenance Guidelines - Upon completion of floor, send the Aacer Floor Maintenance Guide to the owner.  This guide will explain the proper HVAC and building maintenance requirements as well as floor cleaning and servicing guidelines to assure proper floor performance and longevity.

  - WORKING CONDITIONS

- The wood flooring and its components specified herein shall not be delivered or installed until all wet trades and overhead work is completed.  This includes all masonry, painting, plaster, tile, marble, and terrazzo, as well as all overhead mechanical trades.  The building shall be fully enclosed and weather tight and all permanent windows and doorways shall be installed.  Permanent heating and air conditioning shall be installed and working in accordance with building occupation requirements.

- The concrete substrate shall be determined fully cured by industry standards.  It shall be free of all foreign materials and broom cleaned when turned over to the floor installer.  Permanent HVAC units for the building shall have been operating a minimum of one week prior to the floor installation start up.

- During and after installation, building HVAC systems shall maintain a temperature and humidity range compatible with the expected high and low moisture content range of the flooring. The floor installer, based on the building’s HVAC control and geographical situation, shall determine this range.

- Flooring must be stored on site in a dry, well ventilated area while acclimating to site conditions. Moisture content of wood shall be consistent with the ambient conditions of the building as it will be maintained when occupied.

  - WARRANTY AND DISCLAIMER

- Aacer Flooring of Peshtigo, WI hereby warrants the materials it has supplied to be free from manufacturing defects for a period of one year.  This warranty is in lieu of and excludes all other warranties expressed or implied including any implied warranties of merchant ability or fitness for a particular purpose.

- During the warranty period, the floor shall not be recoated without the approval of the flooring contractor.

- The jobsite documentation forms by the flooring contractor shall become a part of the warranty and both the owner and flooring contractor shall retain record of said forms as a permanent reference for any abrogation.

- Flooring contractor warrants the install of the floor systems to be free from defects in materials and workmanship for a period of one year.

- Notification of claim shall be made within 30 days of discovery.

- It is the policy of Aacer Flooring to continuously improve its line of products. Therefore, Aacer Flooring reserves the right to change, modify, or discontinue systems, specifications and accessories of all products at any time without notice or obligation to purchaser.

PART 2 PRODUCTS

  - MATERIALS

- Vapor Barrier – 6 mil polyethylene

- Resilient Pad-

- Aacer EcoDIN Pad 7/16”

  - Optional Size (Specify or Delete)  ½” (12mm) TriPower™ Pad or ½” Performance Foam

- Subfloor –

- Pre-engineered panels as manufactured by Aacer Flooring,  (Aacer Panels), consisting of 1” x 6” and 1” x 4” (nominal) Gym Grade Hemlock, Spruce, Fir or Pine, KD S4S, assembled in to interlocking panel assemblies with pads factory attached for performance requirements

- FSC® Certified (Specify or Delete) Subfloor must be certified by the Forest Stewardship Council™

- Flooring –

- 25/32” x 2-1/4” 2nd and Better grade northern Hard Maple flooring, TGEM, MFMA grade marked and stamped as manufactured by Aacer Flooring.

- Optional Sizes and Grades (Specify or Delete)

  - 25/32” x 1-1/2” ; 25/32” x 2-1/2”; 33/32” x 2-1/4”,33/32” x 1-1/2”

  - 1st grade, 3rd grade, 3rd and better grade

- FSC® Certified (Specify or Delete) Maple must be certified by the Forest Stewardship Council™

- Expansion Bead (Specify or Delete) Flooring shall include 1/64” Expansion Bead.

- Fasteners –

- Flooring - 2” barbed cleats.

- Anchor Pins – Steel Drive Pins or Hammer Drive Pins with rubber bonded washer.

    - Optional Anchor – 100% recycled polymer sized to accept sub-floor pre-assembled panels with pads “Z Clip”.

- Finish Materials – Any seal and finish approved by the MFMA.

- Game Lines – Compatible with finish colors as specified by layout design.

- Wall Base - Heavy duty, molded, vented cove base with pre-molded outside corners.

PART 3 - EXECUTION

3.1. PRE-INSTALLATION INSPECTION

- Floor installer shall verify slab tolerance of concrete and report any needed corrections to general contractor.

- Room shall be broom cleaned and free of any foreign debris.

- Floor installer shall document site and working conditions prior to and during installation.  This documentation shall become a part of any warranty and may or may not affect fulfillment of any warranty.

  - INSTALLATION

- SUBFLOOR –

- Cover entire slab with 6 mil polyethylene, sealing and lapping joints a minimum of 6”.

- Ultra-Flex™ DC sub-floor panels shall be laid perpendicular to the finished flooring using a brick pattern. Attach panels using concrete anchors or anchoring Z clips, leaving 8” spacing between panels.  Use an 8” block to insure proper spacing.  If 3rd Grade flooring is going to be used, the spacing needs to be changed down to 6” between the panels.

- Ultra-Flex™ sub-floor panel frames shall be secured together using 1-1/2" staples, 2 per end tab. Allow ¼” spacing on tab ends.

- Install finish flooring parallel with main playing court by power nailing every 10" to 12" on center.

- Provide 1-1/2” expansion voids at perimeter and all vertical obstructions.  Install solid blocking at doorways, under bleachers in the stacked position, and below portable goals.

- MAPLE FLOORING

- Install Aacer maple flooring parallel with the long dimension of room.  Flooring shall be power nailed every 10” to 12” O.C. with all end joints properly driven tight.

- Expansion joints may be required between flooring strips intermittently throughout the floor.  Requirements will be determined by site and geographical conditions.

- Provide a minimum 1-1/2” expansion void at all walls and permanent obstructions.

  - FINISHING

- FLOOR SANDING

- Machine sand entire floor with multiple grit papers to a smooth and uniform surface, free from edger marks and drum drops.

- Remove all sanding dust and lint from entire surface by vacuum and/or tack.

- FINISHING AND GAME LINES

- Inspect entire floor to be sure surface is ready to accept seal and finish.  Floor should be free from dust and debris.

- Apply (2) coats of approved seal and (2) coats of approved finish per manufacturer’s label instructions.

- Floor shall be buffed, cleaned, and tacked between coats.

- Apply game lines and logos as indicated by drawings between seal and finish coats.  Paint shall be compatible with finish.

- BASE INSTALLATION

- Install vent cove base with cove base adhesive and/or mechanical attachment to wall.  Use pre-molded outside corners and mitered inside corners.

  - CLEANUP

- CLEANUP

- Remove excess debris and waste material from the work area.

- General Contractor shall lock floor area after floor is finished to allow proper curing time. If general contractor or owner requires use of gym before proper curing time, they shall protect the floor by covering with non-marring Kraft paper.

END OF SECTION 09642

Ultra-Flex™ Spec sheet

June, 2012

Revision level A