SPECIFICATION:  
RECLAIM I, Fixed Resilient System

AacerFlooring.com

Sales@AacerFlooring.com

877-582-1181

SECTION 09642-Wood Gymnasium Flooring

1.1 SCOPE OF THE WORK

A.	Flooring contractor shall provide all labor, materials, equipment, service, etc., necessary for  removal of designated components of existing floor system and the proper installation of new hardwood athletic floor system.

B.	Designated components of existing floor system to be removed shall include wall base, equipment cover plates, wood flooring and upper subfloor layer.

C.	Removal of existing floor components shall be completed in a manner resulting in the least destruction of supporting sleepers which shall remain in place for support and attachment of new floor system components.

D.	Flooring contractor shall haul debris from previous floor system installation from job site and dispose of safely and legally.

E.	New floor system installation shall include areas below existing bleachers, which shall be detached, moved, and reattached by others as authorized by bleacher manufacturer.

  - REFERENCES

- FSC – Forest Stewardship Council™

- MFMA – Maple Flooring Manufacturers Association

  - QUALITY ASSURANCE

- Manufacturer

- Manufacturer of resilient flooring shall be a firm specializing in manufacturing products specified in this section.

- Basis of design shall be “ReClaim I” sports floor system as provided by Aacer Flooring.

- Materials other than those listed must be approved 10 days prior by written addendum.

Materials from non-approved manufacturers will not be accepted.

- Installer

- The installation of the floor system described in these specifications shall be completed by a firm familiar with the requirements of the system specified and fully experienced in procedures required for installing athletic flooring manufactured by Aacer Flooring.

- Installer shall be liable for all matters related to installation for a period of one year after the floor has been substantially installed and completed.

- Installer must have Aacer installation accreditation.

  - Optional: MFMA accreditation.(Specify or Delete)

- Construction and Performance

1.	Athletic floor system must provide the following project requirements:

  - Floor system shall be specifically designed to incorporate existing sleepers of previous floor.

  - Floor system shall have been tested by a Scientific Body Member of the International Association of Sports Surface Sciences (ISSS)  and certified as conforming to the MFMA-PUR athletic floor performance standard when evaluated over rigid fixed sleepers.

  - Floor surface of newly installed floor system shall provide finished profile height within 1/4" of existing floor surface when removing previous flooring and nominal 3/4" upper subfloor layer.

  - SUBMITTALS

- Specification - Submit Aacer Flooring specification sheets and shop drawings as required.

- Sample - Submit required number of samples of the specified system as requested by the owner/architect.

- Maintenance Guidelines - Upon completion of floor, send the Aacer Floor Maintenance Guide to the owner. This guide will explain the proper HVAC and building maintenance requirements as well as floor cleaning and servicing guidelines to assure proper floor performance and longevity.

  - WORKING CONDITIONS

- The wood flooring and its components specified herein shall not be delivered or installed until all wet trades and overhead work is completed. This includes all masonry, painting, plaster, tile, marble, and terrazzo, as well as all overhead mechanical trades. The building shall be fully enclosed and weather tight and all permanent windows and doorways shall be installed.

- The concrete substrate shall be determined fully cured by industry standards and materials shall not be stored at the installation location unless the in-slab relative humidity level for the concrete slab is 85% or lower before installation.

- Permanent HVAC units for the building shall have been operating a minimum of one week prior to the floor installation start up.

- During and after installation, the H.V.A.C. system should be complete, operational, and conditioning air to be within specifications (55/75 degrees Fahrenheit (13-27 degrees Celsius) with relative humidity between 35/50 percent) or to conditions expected following installation and during occupancy.

- Flooring must be stored on site in a dry, well-ventilated area while acclimating to site conditions. Moisture content of wood shall be consistent with the ambient conditions of the building as it will be maintained when occupied.

  - WARRANTY AND DISCLAIMER

- Aacer Flooring of Peshtigo, WI hereby warrants the materials it has supplied to be free from manufacturing defects for a period of one year. This warranty is in lieu of and excludes all other warranties expressed or implied including any implied warranties of merchant ability or fitness for a particular purpose. Guarantee shall not cover damage caused in whole or in part by casualty, ordinary wear and tear, abuse, use for which material is not designed, faulty construction of the building, settlement of the building walls, failure of the other contractors to adhere to specifications, separation of the concrete slab and excessive dryness or excessive moisture from humidity, spillage, migration through the slab or wall, or any other source.

- During the warranty period, the floor shall not be recoated without the approval of the flooring contractor.

- The jobsite documentation forms by the flooring contractor shall become a part of the warranty and both the owner and flooring contractor shall retain record of said forms as a permanent reference for any abrogation.

- Flooring contractor warrants the install of the floor systems to be free from defects in materials and workmanship for a period of one year.

- Notification of claim shall be made within 30 days of discovery.

- In the event of breach of any warranty, the liability of Aacer Flooring shall be limited to repairing or replacing ReClaim I material and system components supplied by Aacer Flooring and proven to be defective in manufacture, and shall not include any other damages, either direct or consequential.

- It is the policy of Aacer Flooring to continuously improve its line of products. Therefore, Aacer Flooring reserves the right to change, modify, or discontinue systems, specifications, and accessories of all products at any time without notice or obligation to purchaser.

PART 2 PRODUCTS

2.1. MATERIALS

- Subfloor –

- Two Layers of 11/32” x 4’ x 8’ (9mm x 1219mm x 2438mm) APA rated, Exposure 1 plywood sheathing.

- FSC® Certified Subfloor (Specify or Delete) - Subfloor must be certified by the Forest Stewardship Council™

- Resilient Pads –

1.    VERT resilient pads supported by patented 16-gauge steel cushion support plates.

- Flooring –

- 25/32” (20mm) x 2-1/4” (57mm) 2nd and Better grade northern Hard Maple flooring, TGEM, MFMA grade marked and stamped as manufactured by Aacer Flooring.

- Optional Sizes and Grades (Specify or Delete)

  - 25/32” (20mm) x 1-1/2” (38mm), 33/32” (26mm) x 2-1/4” (57mm), 33/32” x 1-1/2”

  - 1st grade, 3rd grade, 3rd and better grade

- FSC® Certified Flooring (Specify or Delete) – Maple flooring shall be certified by the Forest Stewardship Council™

- Expansion Bead (Specify or Delete) – Maple flooring shall include 1/64” (0.4mm) expansion bead.

- Fasteners –

- Flooring – 1-1/2” (38mm) barbed cleats or staples.

- Subfloor Panels– 3/4” (16mm) coated staples and PL400 or equivalent construction adhesive.

- Subfloor Anchors – !-1/2” (38mm) collared coarse wood screws.

- Steel Cushion Support Plates – 1-1/4” (32mm) wood screws.

- Finish Materials – MFMA approved seal and finish

- Game Lines – Compatible with finish and as specified by layout design

- Wall Base - Heavy duty, molded, vented cove base with pre-molded outside corners.

PART 3 - EXECUTION

3.1. PRE-INSTALLATION INSPECTION

- Floor installer shall inspect integrity of existing sleepers and soundly attach new sleepers to replace those with questionable integrity.

- Floor installer shall document site and working conditions prior to and during installation.  This documentation shall become a part of any warranty and may or may not affect fulfillment of any warranty.

3.2. INSTALLATION

- SUBFLOOR –

- Attach each steel cushion support plate to sleepers with four screws, space 16” (406mm) on center starting with edge of first plate adjacent to wall.

- Install lower subfloor layer parallel to sleepers with panel ends centered over cushion plates. Offset panels 48” (1219mm) in adjacent rows to create staggered brick pattern with ends and side edges spaced nominally 1/4" (6mm). Space panels 2” (51mm) from walls and all vertical obstructions.

- Install upper subfloor layer diagonally to lower layer with panels offset 48” (1219mm) in adjacent rows to create staggered brick pattern, and with side and end edges spaced nominally 1/4" (6mm). Attach each upper panel with application of subfloor adhesive applied in a box-x pattern prior to placement. Space panels 2” (51mm) from walls and all vertical obstructions. Secure to lower plywood layer with subfloor staples applied 6” (152mm) on center along all plywood edges and 12” (305mm) on center throughout interior of each panel.

- Attach completed subfloor layer to existing sleepers with collared wood screws spaced 32” on center along each sleeper row. Offset anchors from plate cushion locations by nominal 8” (203mm) and offset anchors in adjacent sleeper rows to create diamond anchorage pattern. Use special drill bit arrangement to create desired anchor pockets for installation of collared wood screws.

- Install solid blocking at doorways, under bleachers in the stacked position, and below portable goals.

- Provide 1-1/2” (40mm) to 2” (51mm) expansion voids at perimeter and all vertical obstructions.

- MAPLE FLOORING

- Install Aacer maple flooring with fasteners applied nominally 10”-12” (254mm – 305mm) on center with all end joints properly driven tight.

- Install intermediate expansion spacing if required based on anticipated flooring growth.

- Provide 2” (51mm) expansion void at all walls and permanent obstructions.

  - FINISHING

- FLOOR SANDING

- Machine sand entire floor with multiple grit papers to a smooth and uniform surface, free from edger marks and drum drops.

- Remove all sanding dust and lint from entire surface by vacuum and/or tack.

- FINISHING AND GAME LINES

- Inspect entire floor to be sure surface is ready to accept seal and finish. Floor should be free from dust and debris.

- Apply (2) coats of approved seal and (2) coats of approved finish per manufacturer’s label instructions.

- Floor shall be buffed, cleaned, and tacked between coats.

- Apply game lines and logos as indicated by drawings between seal and finish coats.  Paint shall be compatible with finish.

- BASE INSTALLATION

- Install vent cove base with cove base adhesive and/or mechanical attachment to wall.  Use pre-molded outside corners and mitered inside corners.

  - CLEANUP

- CLEANUP

- Remove excess debris and waste material from the work area.

- Floor area shall be locked after floor is finished to allow proper curing time. If owner requires use of gym before proper curing time, they shall protect the floor by covering with non-marring Kraft paper.

END OF SECTION 09642

ReClaim I Specification

Revision Level A 2020

Contact your Regional Sales Manager (1-877-582-1181) or the local Aacer Flooring Authorized Dealer for more information.